| CMGT 235 – Electrical and Mechanical Systems |  |  |
| --- | --- | --- |
| Discussion No. 23 | Unit 3 - Electrical Systems | Fall 2019 |

Voltage Drop

Example #5 - Temporary Job Site Light, Tripod, Corded (AC), Lumens 8000, Number of Lamp Heads 1

| Gauge/Conductor | Ohms/kFT | Length (L) ft |
| --- | --- | --- |
| 18/3 |  |  |
| 16/3 |  |  |
| 14/3 |  |  |
| 12/3 |  |  |

Example #6

A single-phase, 240-volt air-conditioner is being installed for a small commercial building. The nameplate reads: “Minimum Circuit Ampacity 40 Amperes.” The circuit originates at the main panel located 125 ft from the air-conditioner unit.

- Determine the maximum voltage drop of the line recommended by the NEC?

- What is the minimum size THWN CU conductors required and where in the NEC do you find this?

- Determine the voltage drop due to the conductors. Does it meet the code requirement? If not, what should be done to meet the code?