LEED for Neighborhood Development (LEED ND)

Quiz #2 – Smart Location and Linkage (SLL)

- Which of these is a preferable site location for a LEED for Neighborhood Development projects?

- Infill, previously undeveloped, sites adjacent to existing development

- Infill, previously developed, sites adjacent to existing development

- Previously developed, adjacent to previously undeveloped, within previously existing developments

- Sites that are 50% previously undeveloped and 50% previously developed

- SSL Prerequisite Smart Location requires all projects to locate the project on a site served by existing

- multimodal transportation

- parks and nature trails

- bike paths

- water and wastewater infrastructure

- To meet the requirement for SLL prerequisite Smart Location, Option 3. Transit Corridor, projects with multiple transit types (bus, streetcar, rail, or ferry) must have what minimum daily transit service?

- 60 Weekday trips and 40 weekend trips

- 24 Weekday trips and 6 weekend trips

- 60 Weekday trips

- 320 Weekday trips and 200 weekend trips

- How is connectivity measured?

- Gross floor area divided by total project area

- Dwelling units per acre

- Intersections per square mile

- Dwellings units per transit trips

- What units are used to measure a circulation network?

- Lineal feet

- Square Miles

- Square feet

- Acres

- Which of these organizations can be used to determine the presence of imperiled species and ecological communities?

- ASHRAE

- FEMA

- State Fish and Wildlife Agencies

- Conservation Resources Services

- Which of these individuals could help a project to create and implement a habitat conservation plan?

- Arborist to determine the local wildlife and habitat

- Biologist to investigate ecologically endangered areas

- Geologist to identify the source of contamination present in a brownfield

- Hydrologist to conduct a soil survey of potential prime farmland area

- Which of these would meet the requirements for SLL Prerequisite Wetland and Water Body Conservation?

- land within 40 feet of wetlands, and land within 75 feet of water bodies

- land within 40 feet of wetlands, and land within 100 feet of water bodies

- land within 50 feet of wetlands, and land within 100 feet of water bodies

- land within 50 feet of wetlands, and land within 75 feet of water bodies

- Minor construction can occur to enhance appreciation of them in which buffer areas?

- Floodplains

- Wetland and Water Body

- Lakes

- Streams

- Which of these is a farmland protection program administered by government?

- Transfer of Development Rights (TDR)

- Modified Rational Method (MRM)

- FEMA

- Conservation Resource Services (CRS)

- A project in a metropolitan statistical area with a population of 400,000 people has 100 dwelling units on 10 acres (4 hectares) of buildable residential land. It will include a 0.1-acre (0.04-hectare) on-site community garden. The soil survey shows that the project’s development footprint encroaches on 2 acres of prime soil. Determine the required mitigation land area for a project that has a mitigation ratio of 1.5 acres.

- 3.0 acres

- 2.9 acres

- 10 acres

- 3.1 acres

- Which of these is the referenced standard for identifying prime farmland?

- Conservation International

- Department of Agriculture

- NRCS soil survey

- FEMA

- A project has several buildings located on previously developed land within a flood hazard area. Which of these design standards would meet the requirement for SLL Credit Floodplain Avoidance?

- National Flood Insurance Program (NFPI)

- ASHRAE

- Federal Emergency Management Agency (FEMA)

- Difficult Development Area (DDA)

- Which of this site locations would help a project to achieve the most points possible for SLL Credit Smart Locations?

- a previously developed site that is not an adjacent site or infill site

- an adjacent site that is also a previously developed site

- an infill site that is not a previously developed site

- an infill site that is also a previously developed site

- To earn the SLL Credit Brownfield Remediation,

- the entire project site must be a Brownfield.

- the entire project site does not need to be a brownfield.

- a previously remediated brownfield must be on the project site.

- remediation of contaminants in existing buildings, like asbestos or debris found on site, qualifies.

- A development project includes a 10,000-ft2 office building with 40 full-time employees and 10 peak visitors. How many long-term bicycle storage spaces are required for SSL Credit Bicycle Facilities?

- 4 storage spaces

- 0.25 storage spaces

- 2 storage spaces

- 1 storage space

- What is the maximum allowable street speed to be included in a bicycle network?

- 30 mph

- 25 mph

- 20 mph

- 15 mph

- To be considered as part of a bicycle network off-street bicycle paths or trails for a two-way path must be at least how wide?

- 3 feet

- 5 feet

- 6 feet

- 8 feet

- If a project has 300 dwelling units and wants to earn the Smart Location and Linkage (SLL) credit Housing and Jobs Proximity, Option 2. Project with Residential Component, how many full-time equivalent jobs must be within the ½-mile walking distance of the project’s geographic center?

- 60

- 90

- 150

- 300

- For SSL Credit Steep Slope Protection, the restoration areas must use

- Native and adapted plants

- Turf

- Artificial turf

- Temporary seeding

- For SSL Credit Steep Slope Protection, what percentage of previously developed areas of slope greater that 40% must be restored?

- None

- 40%

- 60%

- 100%

- To achieve the requirements for SSL Credit Long-Term Conservation Management of Habitat or Wetlands and Water Bodies projects must create and commit to implementing a management plan for existing or recently restored on-site native habitats, water bodies, or wetlands and their buffers, and create a guaranteed funding source for management for how long?

- ten years

- five years

- two years

- one year