LEED for Neighborhood Development Power Jam Study

Quiz #2 – Smart Location and Linkage (SLL)

- To meet the requirement for SLL prerequisite Smart Location, Option 3. Transit Corridor, projects with multiple transit types (bus, streetcar, rail, or ferry) must have what minimum daily transit service?

- 24 Weekday trips and 6 weekend trips

- 60 Weekday trips and 40 weekend trips

- 60 Weekday trips

- 320 Weekday trips and 200 weekend trips

- Which of these is a farmland protection program administered by government?

- Transfer of Development Rights (TDR)

- Modified Rational Method (MRM)

- FEMA

- Conservation Resource Services (CRS)

- Which of this site locations would help a project to achieve the most points possible for SLL Credit Smart Locations?

- a previously developed site that is not an adjacent site or infill site

- an adjacent site that is also a previously developed site

- an infill site that is not a previously developed site

- an infill site that is also a previously developed site

- What is the maximum allowable street speed to be included in a bicycle network?

- 15 mph

- 20 mph

- 25 mph

- 30 mph

- To be considered as part of a bicycle network off-street bicycle paths or trails for a two-way path must be at least how wide?

- 3 feet

- 5 feet

- 6 feet

- 8 feet

- A project has several buildings located on previously developed land within a flood hazard area. Which of these design standards would meet the requirement for SLL Credit Floodplain Avoidance?

- National Flood Insurance Program (NFPI)

- ASHRAE

- Federal Emergency Management Agency (FEMA)

- Difficult Development Area (DDA)

- SSL Prerequisite Smart Location requires all projects to locate the project on a site served by existing

- multimodal transportation

- parks and nature trails

- bike paths

- water and wastewater infrastructure

LEED for Neighborhood Development Power Jam Study

Quiz #2 – Smart Location and Linkage (SLL)

- To meet the requirement for SLL prerequisite Smart Location, Option 3. Transit Corridor, projects with multiple transit types (bus, streetcar, rail, or ferry) must have what minimum daily transit service?

- 24 Weekday trips and 6 weekend trips

- 60 Weekday trips and 40 weekend trips

- 60 Weekday trips

- 320 Weekday trips and 200 weekend trips

- Which of these is a farmland protection program administered by government?

- Transfer of Development Rights (TDR)

- Modified Rational Method (MRM)

- FEMA

- Conservation Resource Services (CRS)

- Which of this site locations would help a project to achieve the most points possible for SLL Credit Smart Locations?

- a previously developed site that is not an adjacent site or infill site

- an adjacent site that is also a previously developed site

- an infill site that is not a previously developed site

- an infill site that is also a previously developed site

- What is the maximum allowable street speed to be included in a bicycle network?

- 15 mph

- 20 mph

- 25 mph

- 30 mph

- To be considered as part of a bicycle network off-street bicycle paths or trails for a two-way path must be at least how wide?

- 3 feet

- 5 feet

- 6 feet

- 8 feet

- A project has several buildings located on previously developed land within a flood hazard area. Which of these design standards would meet the requirement for SLL Credit Floodplain Avoidance?

- National Flood Insurance Program (NFPI)

- ASHRAE

- Federal Emergency Management Agency (FEMA)

- Difficult Development Area (DDA)

- SSL Prerequisite Smart Location requires all projects to locate the project on a site served by existing

- multimodal transportation

- parks and nature trails

- bike paths

- water and wastewater infrastructure